Complete Procedure

Install Linpac on a Raspberry Pi

with a DigiRig Soundcard and Icom ID-4100

Keyboard-to-keyboard AX.25 packet radio (1200-baud analog FM)

This procedure installs and configures Linpac for keyboard-to-keyboard packet communication on a Raspberry Pi, using a DigiRig USB soundcard interface and an Icom ID-4100 (ID-4100A / ID-4100E) as the analog FM radio.

You need a valid amateur radio license and must identify as required in your country.

# How the pieces fit together

Keyboard  →  Linpac  →  Linux AX.25  →  Direwolf (software TNC)

↓

DigiRig USB sound + RTS PTT

↓

Icom ID-4100 analog FM (not D-STAR)

↓

RF to the other station

Linpac is only a terminal. It does not talk to the DigiRig by itself. Direwolf turns the DigiRig into a 1200-baud AFSK modem. Linux AX.25 is the packet protocol stack Linpac uses.

| Software | Role |
| --- | --- |
| Raspberry Pi OS | Host OS |
| alsa-utils | Identify and set DigiRig audio levels |
| Direwolf | Software TNC / 1200-baud AFSK modem |
| libax25, ax25-tools, ax25-apps | Kernel AX.25 stack, kissattach, axcall, axlisten |
| Linpac | Keyboard-to-keyboard packet terminal |
| socat (optional) | More reliable KISS pty than Direwolf -p |

Do not use D-STAR (DV), VARA, or Winlink for this procedure. Those are different modes. Linpac is analog 1200-baud AX.25 packet on FM.

# 1. Hardware

Raspberry Pi 3B+, 4, or 5 with power supply, HDMI or SSH, and keyboard

DigiRig Mobile (recommended: Silicon Labs CP210x serial port for RTS PTT)

DigiRig ICOM RJ-45 cable (front mic + rear speaker)

USB-C cable: DigiRig to Pi

Icom ID-4100A / ID-4100E

Antenna and radio power

If you have a DigiRig Lite (no COM port), PTT is VOX on the right audio channel. See the Lite note at the end of section 6.

## Cable connections (ID-4100 + DigiRig ICOM RJ-45)

Unplug the hand mic from the front of the ID-4100.

Plug the cable RJ-45 into the front mic jack.

Plug the cable 3.5 mm TRS into the rear speaker jack (the larger jack). Leave the small 2.5 mm DATA jack empty. That DATA jack is for D-STAR cloning/GPS, not analog packet with this cable.

Plug the 4-pin (TRRS) end into the DigiRig socket labeled AUDIO. Nothing goes into the DigiRig serial socket when using this Icom cable.

USB-C from DigiRig to a Pi USB port.

Power on the radio, then the Pi.

# 2. ID-4100 radio settings

Packet via DigiRig is analog FM, not D-STAR.

| Setting | Value | How |
| --- | --- | --- |
| Operating mode | FM (not DV, not FM-N) | Push [MODE] until FM |
| D-STAR / DR | Off | Do not use the DR screen |
| Duplex | OFF (simplex) | Quick menu / DUP |
| Tone / TSQL / DTCS | OFF unless local packet uses CTCSS | [QUICK] → TONE |
| GPS TX Mode | OFF | [MENU] → GPS → GPS TX Mode → OFF |
| PTT Lock | OFF | [MENU] → Function → PTT Lock |
| MIC Gain | 2 (default; later 1–3 if overdriven) | [MENU] → Function → MIC Gain |
| PRIO / priority watch | OFF | [QUICK] → PRIO Watch OFF |
| TX power | LOW at first | Front panel power |
| Volume | About 50% (10–12 o’clock) | [VOL] — this is the audio DigiRig hears |
| Squelch | Fully open (counterclockwise) | Direwolf has its own DCD; closed squelch can hide packet audio |
| Frequency | Local 2 m packet simplex | [V/M] to VFO, then set frequency |

Typical US 2 m packet simplex frequencies (confirm your local band plan): 145.010, 145.030, 145.050, 145.070, 145.090 MHz. Many Winlink RMS stations use 145.670 — that is a different service; use a quiet simplex channel for keyboard chat.

Both stations must be on the same frequency, FM, simplex, and the same tone setting.

# 3. Raspberry Pi OS

Use current Raspberry Pi OS (Bookworm or later). 64-bit is fine if kernel and userland are both 64-bit. Do not mix a 64-bit kernel with 32-bit userland.

sudo apt update

sudo apt full-upgrade -y

sudo reboot

After reboot, install packages:

sudo apt install -y \

git cmake build-essential \

libasound2-dev libudev-dev \

alsa-utils usbutils \

libax25 ax25-apps ax25-tools \

libncurses-dev automake autoconf libtool \

socat

Add your user to the serial and audio groups (replace pi if your username differs):

sudo usermod -aG dialout,audio $USER

Log out and back in (or reboot) so group membership takes effect.

# 4. Confirm the DigiRig on the Pi

Plug the DigiRig in, then:

lsusb

aplay -l

arecord -l

ls -l /dev/serial/by-id

You should see something like:

USB audio: C-Media / USB PnP Sound Device / Device

Serial (Mobile only): Silicon Labs CP210x → /dev/ttyUSB0

Note the card number from aplay -l. Example:

card 2: Device [USB PnP Sound Device], device 0: USB Audio [USB Audio]

That card is plughw:2,0 (or by name plughw:Device,0).

Also note the serial path. Prefer the stable by-id name:

/dev/serial/by-id/usb-Silicon_Labs_CP2102N_USB_to_UART_Bridge_Controller_........-if00-port0

If you only have /dev/ttyUSB0, that is fine as long as nothing else is plugged in.

Set levels (replace 2 with your card number):

alsamixer -c 2

F6 to select the DigiRig card

Playback (PCM / Speaker): start around 40–60%

Capture (Mic): around 70%, not muted

Disable Auto-Gain if it appears

Icom mic inputs are sensitive. If the radio overdrives or PTT is flaky, lower playback, not raise it.

# 5. Install Direwolf from source

The distro package is often older. Build current Direwolf:

cd ~

git clone https://github.com/wb2osz/direwolf.git

cd direwolf

mkdir build && cd build

cmake ..

make -j$(nproc)

sudo make install

make install-conf

make install-conf copies direwolf.conf into your home directory (~/direwolf.conf).

# 6. Configure Direwolf

Edit ~/direwolf.conf. Replace YOURCALL, the sound card, and the serial path.

ADEVICE  plughw:2,0

ACHANNELS 1

CHANNEL 0

MYCALL YOURCALL-1

MODEM 1200

PTT /dev/ttyUSB0 RTS

# Timing for a mobile FM radio (values are 10 ms units)

DWAIT 0

TXDELAY 30

TXTAIL 10

AGWPORT 8000

KISSPORT 8001

Notes:

ADEVICE must match the DigiRig card from aplay -l. Using the name is more stable: ADEVICE plughw:Device,0

DigiRig Mobile PTT is RTS, not DTR

YOURCALL-1 is the on-air packet identity. Use a unique SSID on each radio if both ends share one callsign (example: N0CALL-1 and N0CALL-2)

## DigiRig Lite (no serial port)

Omit the PTT line and let the Lite’s VOX circuit key the radio from the right audio channel, or keep levels conservative so VOX is reliable. Confirm the Lite LED keys when Direwolf transmits. If it never keys, the Lite may need the right-channel PTT path; in that case use DigiRig Mobile instead.

Test Direwolf by itself (leave this terminal running):

direwolf -t 0

-t 0 turns off color (cleaner over SSH). You should see the sound card open and, when the radio hears packet or noise, an audio level meter. Tune a busy APRS channel (US: 144.390 FM) briefly to confirm decode. Then go back to your packet simplex frequency.

PTT test: in another terminal, while Direwolf is running with default APRS beacon disabled, you can send a keyboard test later from Linpac. You can also watch the DigiRig PTT LED when a packet goes out.

Comment out or leave PBEACON unused so you do not beacon APRS on a packet chat frequency.

# 7. Configure Linux AX.25

Edit /etc/ax25/axports (create the file if missing). No blank lines in the file.

sudo nano /etc/ax25/axports

# name  callsign     speed  paclen  window  description

radio   YOURCALL-1   19200  255     2       2m 1200 packet ID-4100

radio is the port name Linpac will use

callsign should match Direwolf MYCALL

speed is the KISS serial rate to Direwolf, not the on-air baud. 19200 is fine

paclen 255 and window 2 are reasonable for VHF simplex

Make axlisten usable from a normal user (Debian names it axlisten; Linpac often looks for listen):

sudo chmod u+s /usr/bin/axlisten

sudo ln -sf /usr/bin/axlisten /usr/local/bin/listen

sudo mkdir -p /var/ax25/mail /var/ax25/mheard

sudo chown "$USER" /var/ax25/mail

# 8. Install Linpac from source

Do not use the old Raspbian linpac package if it is offered. Build the develop branch.

cd ~

git clone https://git.code.sf.net/p/linpac/linpac linpac

cd linpac

git checkout develop

sudo apt install -y automake autoconf libtool libncurses-dev

autoreconf --install

./configure --prefix=/usr

make -j$(nproc)

sudo make install

If autoreconf fails with an undefined LIBTOOL error:

libtoolize

autoreconf --install

Then configure/make/install again.

Confirm:

which linpac

linpac --help || true

# 9. Start the stack (order matters)

Open three terminals (or use tmux / screen).

## Terminal 1 — Direwolf as a KISS TNC

direwolf -t 0 -p

Wait until you see:

Ready to accept KISS client application on port 8001 ...

Virtual KISS TNC is available on /dev/pts/N

Created symlink /tmp/kisstnc -> /dev/pts/N

Leave this running. If -p later causes kissattach “error setting line discipline”, use the socat method in the troubleshooting section instead.

## Terminal 2 — Attach AX.25

sudo kissattach /tmp/kisstnc radio

sudo kissparms -p radio -t 300 -l 10 -s 100 -r 64

Success looks like:

AX.25 port radio bound to device ax0

Optional monitor in this or another terminal:

sudo axlisten -a -c -t

## Terminal 3 — Linpac

linpac

First run creates ~/LinPac/ and asks:

| Prompt | What to enter |
| --- | --- |
| Callsign without SSID | YOURCALL |
| Home BBS | YOURCALL-1 (your own packet SSID is fine if you have no BBS) |
| Port name | radio (must match /etc/ax25/axports) |
| Digipeaters | Enter (none) |
| Hierarchical address | Your region, e.g. #NM.USA.NOAM — or a placeholder if unsure |

If the wizard already ran and you need to redo it:

rm -rf ~/LinPac

linpac

# 10. Keyboard-to-keyboard operation

Linpac commands start with a colon (:). Anything else is sent to the connected station.

## Direct connected chat (true keyboard-to-keyboard)

Both stations on the same simplex FM frequency, both stacks running. On station A:

:c OTHERCALL-1

Wait for CONNECTED to OTHERCALL-1. Then type normally (no colon) and press Enter. Each line is a packet.

Disconnect:

:d

## Incoming connect

Leave Linpac running on a channel (F1–F8). When the other station connects to YOURCALL-1, the channel shows connected and you type the same way.

Edit ~/LinPac/macro/ctext.mac if you want an automatic greeting, for example:

Hello from YOURCALL-1, type away.

## Unproto / CQ (no connection)

F10 is the unproto channel. Or:

:unsrc YOURCALL-1

:undest CQ

:unproto Anyone on frequency? de YOURCALL-1

The other station will see UI frames in the monitor. Unproto is broadcast; connected :c is the reliable keyboard-to-keyboard QSO.

## Useful keys

| Key / command | Action |
| --- | --- |
| F1–F8 | QSO channels |
| F10 | Unproto / monitor send |
| Alt+X | Quit Linpac |
| :c CALL | Connect |
| :d | Disconnect |
| :port radio | Select AX.25 port |
| :help | Help |

## Quick test without Linpac

axcall radio OTHERCALL-1

If axcall connects, Direwolf, DigiRig, radio, and AX.25 are good; remaining issues are Linpac config.

# 11. Optional start script

Save as ~/start-packet.sh and chmod +x ~/start-packet.sh.

#!/bin/bash

set -e

cd "$HOME"

# Start Direwolf in the background with KISS pty

direwolf -t 0 -p > /tmp/direwolf.log 2>&1 &

echo $! > /tmp/direwolf.pid

sleep 3

if [ ! -e /tmp/kisstnc ]; then

echo "Direwolf did not create /tmp/kisstnc — see /tmp/direwolf.log"

exit 1

fi

sudo kissattach /tmp/kisstnc radio

sudo kissparms -p radio -t 300 -l 10 -s 100 -r 64

echo "AX.25 up. Run: linpac"

echo "Direwolf log: /tmp/direwolf.log"

Stop:

sudo killall kissattach linpac 2>/dev/null || true

kill "$(cat /tmp/direwolf.pid)" 2>/dev/null || true

# 12. Audio and PTT checkout

Open squelch on the ID-4100. Direwolf’s underrun/level line should move.

Brief decode test on 144.390 APRS (US) or a known local packet channel. You should see decoded frames.

Return to your simplex chat frequency.

From Linpac F10 send an unproto. The DigiRig PTT LED should light and the ID-4100 should transmit.

A second receiver (HT) on the same frequency should hear the classic 1200-baud packet burst, not voice, not D-STAR.

Adjust using the table below.

| Symptom | Fix |
| --- | --- |
| Radio does not key; DigiRig LED off | Wrong PTT device, not in dialout, or Lite with no VOX |
| DigiRig LED on, radio does not key | Cable not fully in mic jack; PTT Lock ON; wrong Icom cable |
| Keys but nobody decodes you | Lower alsamixer playback; MIC Gain 1–2; TXDELAY 40 |
| You never decode them | Raise radio volume; raise capture; squelch open; confirm FM not DV |
| Retries / corrupted text | Too much TX audio (ALC/clipping); lower playback |
| Constant transmit | Wrong PTT polarity or serial device; unplug USB and check PTT … RTS |

Direwolf prints Audio level for PLUGHW:... — for received packets, mid-scale (around 50) is healthy. Pegged 100 is too loud.

# 13. Troubleshooting

## kissattach: error setting line discipline

Use TCP KISS + socat instead of direwolf -p.

In ~/direwolf.conf you already have KISSPORT 8001. Start Direwolf without -p:

direwolf -t 0

Then:

socat pty,raw,echo=0,link=/tmp/kisstnc tcp:127.0.0.1:8001 &

sleep 1

sudo kissattach /tmp/kisstnc radio

If kissattach needs the real pty:

sudo kissattach -l "$(readlink -f /tmp/kisstnc)" radio

## No sound card / card number changes after reboot

Use the ALSA name in Direwolf:

ADEVICE plughw:Device,0

List names with aplay -l.

## Permission denied on /dev/ttyUSB0

groups   # must include dialout

sudo usermod -aG dialout $USER

Then log out and back in.

## Linpac monitor empty / crashes on listen

sudo chmod u+s /usr/bin/axlisten

ls -l /usr/local/bin/listen

## Connected but no text, or immediate disconnect

Same frequency and FM on both ends

Different SSIDs (-1 vs -2) if same callsign

axports callsign matches Direwolf MYCALL

Linpac port name is radio

## ID-4100 stays in D-STAR

Push [MODE] to FM. GPS TX Mode OFF. Do not use DR.

# 14. What you should have installed (checklist)

## On the Pi

Raspberry Pi OS updated

alsa-utils, usbutils

Direwolf (built from source)

libax25, ax25-apps, ax25-tools

socat (backup KISS path)

Linpac (built from develop)

User in dialout and audio

~/direwolf.conf pointing at DigiRig audio + RTS PTT

/etc/ax25/axports port named radio

## On the ID-4100

FM simplex, GPS TX off, PTT Lock off

Mic + speaker cable, DATA jack unused

Volume ~50%, squelch open, power LOW

## On the air

Second packet station (another Linpac/Direwolf, or Windows UZ7HO Soundmodem + EasyTerm) on the same frequency

:c OTHERCALL-1 then type without a colon

# References

Direwolf: https://github.com/wb2osz/direwolf

Linpac project: https://sourceforge.net/projects/linpac/

Linpac user manual: https://linpac.sourceforge.net/doc/manual.html

DigiRig Icom RJ-45 cable: https://digirig.net/product/icom-rj45-cable/

Raspberry Pi AX.25 + Direwolf overview: https://www.kevinhooke.com/2022/03/03/revisiting-packet-radio-on-a-raspberry-pi-with-direwolf-part-2-minimal-installation/

Note: The ID-4100 + DigiRig physical hookup (front mic RJ-45 + rear speaker) matches the Delaware County ARES ID-4100 DigiRig cabling used for digital modes. This procedure uses that same cable path for analog 1200-baud AX.25 packet with Linpac.